CE-580

COMPUTATIONAL TECHNIQUES

FOR

FLUID DYNAMICS

HOMEWORK #6

Determination of Best Overrelaxation Parameter for PSOR Method

Taha Yaşar Demir

1881978

# Calculations

Model equations are

Laplace Equation :  ∂2u∂x2+∂2u∂y2=0

Poisson Equation :∂2u∂x2+∂2u∂y2=s

Numerical solutions to above equations are the same, both are solved using so called five point formula

Five-Point Formula :ui+1,j-2ui,j+ui-1,jΔx2+ui,j+1-2ui,j+ui,j-1Δy2=si,j   OΔx2,Δy2

Figure 1: Five Point Stencil

Five-Point formula results in a system of K=(N-2)x(M-2) simultaneous linear algebraic equations and can be solved with direct and iterative methods. In this case we will use Point-Successive Over-Relaxation (PSOR) method which is an iterative method.

PSOR method is derived below

The derivatives can be expressed as

uxx=aui+1,j+bui,j+cui-1,j

uyy=dui,j+1+eui,j+fuij-1

Substituting into the Poisson equation and solving for ui,j

ui,j= -1b+eaui+1,j+cui-1,j+dui,j+1+fuij-1-si,j

Which can be used to estimate ui,jn+1 as an explicit method running in positive I and j diretions

ui,jn+1= -1b+eaui+1,jn+cui-1,jn+1+dui,j+1n+fuij-1n+1-si,jn

Adding  ui,jn-ui,jn to the right-hand side and regrouping gives

ui,jn+1=ui,jn -1b+eaui+1,jn+cui-1,jn+1+dui,j+1n+fuij-1n+1+(b+e)ui,jn-si,jn

Or

ui,jn+1=ui,jn+ωRi,jn+1

Where ω is overrelaxation factor and 1<ω<2

If we assume Δx=Δy=Δ above relation becomes

ui,jn+1=ui,jn+ωRi,jn+1

Ri,jn+1=14ui+1,jn+ui-1,jn+1+ui,j+1n+uij-1n+1+4ui,jn-Δ2si,jn

Problem and Domain

Domain is an 1x1 square with constant division

Figure 2: Square Domain (M=N)

Equation to be solved and used as boundary conditions is

Ux,y=x2-y2

Which satisfies Poisson equation

∂2Ux,y∂x2+∂2Ux,y∂y2=s=∂2x2-y2∂x2+∂2x2-y2∂y2

Taking derivative

∂(2x)∂x+∂(-2y)∂y=s

Taking derivative once again

2-2=s=0

As seen above source term is zero and Laplace’s equation is to be solved with different grid numbers

Boundary Conditions

Boundary conditions is set from the analytical solution on boundary domain as

S1,j=U1,j

SN,j=UN,j

Si,1=Ui,1

Si,N=Ui,N

Where

x1=y1=0.

xN=yN=1.

Error Calculation

Error is calculated as

Error=i=2N-1j=2N-1Ui,j-Si,j (N-2)(N-2)

# Results and Discussion

## Part A

In this part above problem is iterated 20 times with for all values of ω between 1<ω<2 with Δω=0.002

Gird numbers and best ω values are tabulated below

| Grid Number (N) | ω | Error |
| --- | --- | --- |
| 21 | 1.630 | 2.493E-04 |
| 31 | 1.786 | 4.791E-03 |
| 41 | 1.864 | 1.396E-02 |
| 51 | 1.900 | 2.495E-02 |
| 61 | 1.926 | 3.619E-02 |
| 81 | 1.956 | 5.559E-02 |
| 101 | 2.000 | 6.141E-02 |
| Table 1: Best Possible ω values |  |  |

Considering all cases has same boundary conditions and same domain shape, it is evident that best omega values are depend on grid number.

Figure 3: Error w.r.t Omega

Figure 3 shows the error variation with respect to omega, higher relaxation factor does not mean faster convergence. Gird number N=21 gives the best performance since the solution is limited to 20 iteration. Since an explicit solution is used, an information on the boundary marches slowly in domain. With 21 grid nodes, information on the boundary reaches everywhere in domain in 20 iterations, that is why the error is smallest in this case. When grid number is higher, probably internal nodes are not updated according to boundary conditions and introduce errors.

## Part B

In this part, domain has 51x51 grid and it is fixed. Iteration number is also fixed to 1000. Relaxation factors used are

ω=1.6 ;1.7 ;1.8 ;1.9;1.99

Figure 4: Error Variation with Iteration Count

The effect of omega number to error magnitude is shown in figure 4. As previously mentioned, higher omega does not mean faster convergence. When ω=1.99 , solution oversteps and misses the optimum point and fluctuations occur. For w=1.9, there are still small fluctuations and for   ω=1.8 fluctuations are removed. Considering fluctuation presence and final error magnitude, it can be said that best ω is between 1.8 and 1.9 for grid number 51.

# Source Code

program OverRelaxation c..Taha Yaşar Demir / 1881978 c..CE580 - HomeWork #6 PART-A parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e open(11,file="omega.dat") print*, "Enter division number" read*, N w = 1. call grid do while (w.le.2.) call init do k=1,20 ! solution iteration call solution enddo call error_cal call output old_e = e w = w + dw enddo print*, w,e close(11) stop end c----------------------------------------------------------------------- subroutine grid parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e dw = 0.002 dx = 1./(N-1) dy = 1./(N-1) x(1) = 0. y(1) = 0. do i=2,N x(i) = x(i-1) + dx y(i) = y(i-1) + dy enddo return end c----------------------------------------------------------------------- subroutine init parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e do i=1,N do j=1,N s(i,j) = 0. ! Initialize internal points u(i,j) = x(i)**2 - y(j)**2 ! Analytical solution enddo enddo ! boundary conditions do i=1,N s(i,1) = u(i,1) s(i,N) = u(i,N) enddo do j=1,N s(1,j) = u(1,j) s(N,j) = u(N,j) enddo return end c----------------------------------------------------------------------- subroutine solution parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e do i=2,N-1 do j=2,N-1 r(i,j) = 0.25*(s(i+1,j) + s(i-1,j)+ + s(i,j+1) + s(i,j-1) - 4*s(i,j)) s(i,j) = s(i,j) + w*r(i,j) enddo enddo return end c----------------------------------------------------------------------- subroutine error_cal parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e e = 0. ! e : Error do i=2,N-1 do j=2,N-1 e = e + abs(u(i,j)-s(i,j))/((N-2)*(N-2)) enddo enddo return end c----------------------------------------------------------------------- subroutine output parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e write(11,*) w,e if (old_e.lt.e) print*, w,e ! print out best omega value return end

Source Code for Part A

program OverRelaxation c..Taha Yaşar Demir / 1881978 c..CE580 - HomeWork #6 PART-B parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e open(11,file="error.dat") print*, "Enter Omega Value" read*, w call grid call init do k=1,1000 ! solution iteration call solution call error_cal call output(k) enddo close(11) stop end c----------------------------------------------------------------------- subroutine grid parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e N = 51 dx = 1./(N-1) dy = 1./(N-1) x(1) = 0. y(1) = 0. do i=2,N x(i) = x(i-1) + dx y(i) = y(i-1) + dy enddo return end c----------------------------------------------------------------------- subroutine init parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e do i=1,N do j=1,N s(i,j) = 0. ! Initialize internal points u(i,j) = x(i)**2 - y(j)**2 ! Analytical solution enddo enddo ! boundary conditions do i=1,N s(i,1) = u(i,1) s(i,N) = u(i,N) enddo do j=1,N s(1,j) = u(1,j) s(N,j) = u(N,j) enddo return end c----------------------------------------------------------------------- subroutine solution parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e do i=2,N-1 do j=2,N-1 r(i,j) = 0.25*(s(i+1,j) + s(i-1,j)+ + s(i,j+1) + s(i,j-1) - 4*s(i,j)) s(i,j) = s(i,j) + w*r(i,j) enddo enddo return end c----------------------------------------------------------------------- subroutine error_cal parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e e = 0. ! e : Error do i=2,N-1 do j=2,N-1 e = e + abs(u(i,j)-s(i,j))/((N-2)*(N-2)) enddo enddo return end c----------------------------------------------------------------------- subroutine output(iter) parameter(mx=101) common/mk_grid/ dx,dy,x(mx),y(mx),N common/analytic/ u(mx,mx) common/numeric/ s(mx,mx),w,dw,r(mx,mx) common/error/ e,old_e integer iter write(11,*) iter,e return end

Source Code for Part B